Table of Contents

Fish Passage Injury & Mortality
State of the Art

A mechanism-based synthesis of a 255-paper downstream-passage library (1928–2026)
Quantitative claims verified against 37 key papers; see the extraction spreadsheet (‘Confidence’ column)

# 1. Scope & corpus

This synthesis covers 255 publications on downstream fish passage and the injury and mortality it causes — passage through hydropower turbines, pumps, Archimedean screws, weirs, spillways, and bypass/fish-friendly structures (1928–2026). Headline quantitative claims below were checked against the original papers; 37 key empirical studies are marked ‘Verified’ in the companion spreadsheet. Other cell values remain text-mined first-pass estimates.

| Study type | Papers | Contribution |
| --- | --- | --- |
| Lab | 77 | Controlled dose–response: pressure chambers, shear flumes, blade-strike rigs |
| Field | 68 | Real-structure survival, telemetry, Sensor Fish, ecological assessment |
| Numerical | 48 | CFD, blade-strike & particle models, BioPA / bio-hill charts |
| Review | 49 | Syntheses, overviews, meta-analyses |
| Guidelines | 13 | Standards & criteria (NEN 8775, German/Austrian/Dutch/Canadian guidance) |

Mechanism coverage is dominated by barotrauma and shear, with blade strike close behind (counts exceed 255 because most studies address several mechanisms): barotrauma/pressure 87; shear 72; blade strike 52; turbulence 43; entrainment/impingement 36; cavitation 29; gas supersaturation 15; grinding 11.

How big is the problem? The best single estimate is from a global meta-analysis of >275,000 fish across 75 species (Radinger & van Treeck 2021): average hydropower-turbine fish mortality of 22.3% (95% CI 17.5–26.7%) after correcting for handling/catch effects — but with large variation among turbine types, methods and taxa. Older syntheses put direct turbine kill at roughly 3–30% depending on fish length and turbine design (Amaral & Hecker 2011).

# 2. Injury mechanisms

## 2.1 Barotrauma (rapid decompression) — verified landmarks

Barotrauma is the most intensively studied mechanism. The rapid pressure drop near the runner (the ‘nadir’) over-expands swim-bladder gas, causing emboli, swim-bladder rupture, exophthalmia and internal haemorrhage. Susceptibility depends on the change relative to acclimation, captured by the Ratio of Pressure Change (RPC = acclimation pressure ÷ nadir) and its log form (LRP). The PNNL programme (Brown, Carlson, Pflugrath, Stephenson, Colotelo) established ‘simulated turbine passage’ in hyper/hypobaric chambers; Brown et al. (2012) showed injury is driven mainly by Boyle’s-law gas expansion rather than Henry’s-law gas exsolution, and McKinstry et al. (2007) replaced raw mortality with a ‘mortal injury’ endpoint.

| Metric / question | Verified value | Source |
| --- | --- | --- |
| Acclimation depth ceiling (juv. Chinook) | Neutral buoyancy only to ~4.6–11.6 m; deeper = higher risk | Pflugrath 2012 |
| Precautionary pressure threshold | Keep exposure pressure ≥ ~70% of acclimation pressure (Murray cod, silver perch) | Boys 2016 (piecewise) |
| Eggs & larvae | Highly vulnerable; injuries near 100% at low pressure; keep ≥ ~40% surface pressure | Boys 2016 (How low) |
| RPC dose–response (Neotropical) | 50% swim-bladder rupture at RPC ≈ 1.75 (pacu) – 2.2 (piracanjuba) | Kerr 2023 |
| Lamprey tolerance | No injury at 146.2→13.8 kPa, where ~97.5% of juv. Chinook would be injured/killed | Colotelo 2012 |
| American eel tolerance | Resilient at 172 kPa acclimation; barotrauma not a major concern | Pflugrath 2019 |
| Tag effect | Tag presence + LRP are the strongest predictors of mortal injury (tags inflate mortality) | Carlson 2012 |

Two important nuances. First, the field’s workhorse assumption — that barotrauma follows the static Boyle’s law — has been directly challenged: Kerr et al. (2023) show a dynamic (Modified Rayleigh–Plesset) model diverges from Boyle’s law near zero nadir pressure and under liquid tension, implying static chamber thresholds can mis-state real risk. Second, the common shorthand that ‘physostomes (open swim bladder) are more tolerant’ holds for eel and lamprey but is not universal: Zitek et al. (2026) found physostomous cypriniform larvae and juveniles among the most barotrauma-sensitive groups tested, and observed lower mortality under partial-load operation. Susceptibility is therefore species- AND life-stage-specific, not reducible to bladder type alone.

## 2.2 Fluid shear — verified landmarks

Shear injury arises where steep velocity gradients (shear layers, jets, draft tubes, spillway plunge) tear at body surface, eyes, opercula and gills. The exposure metric is the strain rate (s⁻¹), operationalised in the PNNL submerged-jet apparatus (Neitzel 2000). Injury severity rises with jet velocity / strain rate and with fish size and orientation; eye and operculum injuries appear first, and acceleration predicts injury better than velocity alone.

| Finding | Verified value | Source |
| --- | --- | --- |
| Injury onset (juv. Chinook, 93–128 mm) | Opercle injury ~12.2 m/s; eye injury/equilibrium loss ≥16.8 m/s jet | Deng 2005 |
| Minor/major injury onset (submerged jet) | Minor ~15.2 m/s; major ~21.3 m/s | Deng 2010 |
| Mekong species | Injury rose with strain rate to ~1185 s⁻¹; gourami > shark susceptibility | Colotelo 2018 |
| American eel | No injury at strain rate >800 s⁻¹ (immediate or 48 h) — shear-resistant | Pflugrath 2021 |

Comparison trap: strain rate (s⁻¹) is not shear stress (Pa); the two are frequently conflated in secondary literature.

## 2.3 Blade strike — verified landmarks

Blade strike is direct mechanical impact by a runner blade. Mortality rises with strike velocity, fish length, thinner blades, and strike location/orientation; engineering responses (blunt, slanted leading edges) reduce it.

| Finding | Verified value | Source |
| --- | --- | --- |
| Strike severity drivers | Midbody > head > tail; lateral > dorsal/ventral; thinner blades & higher velocity worse | Bevelheimer 2019 |
| Leading-edge geometry (rainbow trout, 10 m/s) | 98% survival at 30° slant vs 26.8% at 90° (L/t=2); 200-mm fish 68% vs 7.9% by strike location | Amaral 2020 |
| Body shape | Anguilliform eel resists strike; laterally-compressed bluegill highly susceptible | Saylor 2019 |
| Surrogacy | Salmonid surrogacy for untested species is unreliable; size & morphology matter | Saylor 2020 |
| Novel runner (white sturgeon) | Higher survival through a thick, slanted fish-friendly runner vs conventional | Zillig 2025 |
| Overall | Blade strike often the primary kill mechanism (3–30%); blunter/slower = safer | Amaral 2011 |

## 2.4 Turbulence, cavitation, grinding, gas supersaturation

- Turbulence — disorients fish and increases strike/shear exposure; usually studied with shear/behaviour (Cada; Odeh).

- Cavitation — vapour-bubble collapse; in this corpus mostly turbine-engineering (Avellan; Escaler) rather than direct fish injury.

- Grinding / abrasion / pinch — fish crushed in narrow gaps (runner–hub, screw–trough); for American eel, the likely dominant injury route alongside strike (Pflugrath 2021).

- Gas supersaturation / GBT — total dissolved gas from spillway plunge; distinct from barotrauma; depth modulates the effect (Morris; Watson).

# 3. Cross-cutting findings

## 3.1 Species & life-stage sensitivity

The evidence base is salmonid-centric: juvenile Chinook and rainbow trout/steelhead dominate (Chinook 57 studies, rainbow 34), with Atlantic salmon, American shad and European/American eel well represented. Tolerance ordering is mechanism-specific: eel and lamprey are highly barotrauma- and (for eel) shear-tolerant (Colotelo 2012; Pflugrath 2019, 2021), yet eel remain vulnerable to blade strike and grinding. The blanket ‘physostomes are tolerant’ rule fails for cypriniform larvae/juveniles, which Zitek (2026) found highly sensitive. Juveniles are by far the most-tested stage (74 studies); eggs and larvae (Boys 2016) are under-studied despite high vulnerability.

## 3.2 Fish size

Larger fish face higher blade-strike probability and mortality (length relative to blade spacing; Amaral 2020, Bevelheimer 2019), while barotrauma scales with swim-bladder physiology and acclimation rather than length. Interpret size mechanism-by-mechanism, not as a single vulnerability axis.

## 3.3 Structure / turbine type

Kaplan turbines are the most-studied structure (62 studies), then spillways (44), Francis (40) and pumps (38). Field comparisons show conventional Kaplan plants can be most harmful (up to ~83% mortality; Mueller 2022), whereas Very Low Head turbines (Tuononen 2020: only 1.16% of entrained fish killed) and purpose-built fish-friendly runners (Zillig 2025; Watson) are markedly safer. Small, well-bypassed projects can reach 77–97% total passage survival (Amaral 2018). Pumps/pumping stations form a distinct sub-literature where strike and shear dominate over barotrauma (van Esch; Pan; Bierschenk).

# 4. Methods landscape

| Approach | Strength | Blind spot / assumption |
| --- | --- | --- |
| Lab (chamber / flume / strike rig) | Clean dose–response for one mechanism | Single-mechanism; assumes acclimation/neutral buoyancy; apparatus ≈ real turbine |
| Field (mark–recapture, telemetry, Sensor Fish) | Integrates all mechanisms at a real structure | Tag/handling burden (Carlson 2012); immediate vs delayed; representativeness |
| Numerical (CFD, blade-strike, BioPA) | Maps the full exposure field; compares designs pre-build | Predicts exposure not biology unless coupled to dose–response; idealized geometry |

The integrative frontier couples CFD exposure distributions (pressure, shear, strike) to lab dose–response via BioPA / bio-hill charts (Richmond 2015; Romero-Gomez) and validates against field tests; Tomanova (2023) updated Francis-turbine mortality formulae (peripheral speed, diameter, fish size; r=0.89 vs observed). Sensor Fish and balloon-tag recovery bridge lab and field by measuring the physical exposure live fish experience.

# 5. Key assumptions & where they are contested

- Static Boyle’s-law barotrauma vs dynamic decompression — challenged by Kerr (2023).

- Neutral buoyancy at acclimation depth — standard, but wild fish at an intake may not be neutrally buoyant, changing RPC (Pflugrath 2012).

- Cross-species surrogacy — salmonid thresholds are widely reused but repeatedly shown not to transfer (Saylor 2020; Pflugrath 2018; Zitek 2026).

- Physostome tolerance as a rule — true for eel/lamprey, false for cypriniform larvae (Zitek 2026).

- Sensor Fish / biomimetic ≈ live fish (Saylor 2021); CFD exposure ≈ biological dose — only via a validated dose–response model.

- Immediate ≈ total mortality — delayed (48–96 h) and tag effects can substantially change the number (Carlson 2012).

# 6. Research gaps & recommendations

- Broaden beyond salmonids: eggs/larvae, eels, lamprey, cyprinids and non-temperate species relative to their exposure.

- Resolve Boyle’s-law vs dynamic barotrauma with rate-varying (not just magnitude-varying) experiments.

- Standardise reporting: pair every threshold with metric, units, species, life stage and acclimation depth (the extraction schema enforces this).

- Strengthen CFD exposure-to-response coupling and independent field validation of BioPA/bio-hill predictions.

- Quantify combined/sequential exposure (strike + shear + barotrauma) rather than single mechanisms.

- Expand delayed-mortality and sub-lethal endpoints beyond immediate survival.

- Build matched-condition comparisons of novel fish-friendly designs (VLH, screw, shaft, Natel/Alden runners) vs conventional turbines.

Provenance: figures attributed to a named study above were read from that paper and are marked ‘Verified’ in the extraction spreadsheet (37 studies). Corpus-level counts come from automated text mining of all 255 PDFs and should be treated as close estimates. Remaining unverified cells in the spreadsheet (Confidence = ‘Mined’) should be checked before citation.